# EDA284 Exam 2020-03-21

John Croft

19930814-7959

1a)

We assume that equal amounts of data are read and written to memory.

Serial phase is always 0,5s.

Parallel phase consists of 16 Gflop.

Arithmetic Intensity is 0.4 Flop/Byte.

16 Gflop / 0.4 Flop/Byte = 40 GB of data to be transferred (one way), or 80GB two ways.

Memory bandwidth for CMP is 16 GB/s.

Results must be written back to memory, halving bandwidth to 8 GB/s.

Peak performance of each core is 0.5 GFlop/s.

For 4 cores, the peak throughput is 4x0.5 GFlop/s = 2 GFlop/s.

8 GB/s bandwidth constitutes 8GB/s x 0.4 Flop/Byte = 3.2 GFlop/s.

Therefore, it is compute-bound in this case.

For 8 cores, peak throughput is 8x0.5 GFlop/s = 4 GFlop/s.

The system is now memory-bound.

For 16 cores, the peak throughput double but the system is still memory-bound, and no further performance can be achieved.

| Cores | Peak throughput | Texe | Notes |
| --- | --- | --- | --- |
| 4 | 2 GFlops/s | 0.5s + 16 Gflop / 2 GFlop/s = 8,5s | Compute-bound |
| 8 | 4 GFlops/s | 0.5s + 16 Gflop / 3.2 GFlop/s = 5,5s | Memory-bound |
| 16 | 8 GFlops/s | 5,5s | Memory-bound |

1b)

Serial phase is still fixed at 0,5s.

Memory must first be transferred to device memory ie. 16/0.4 = 40GB/16GB/s = 2,5s, and back again for a total of 5s.

Data must be written to and from device memory as well, halving available bandwidth 128/2 = 64GB/s.

64GB/s of bandwidth constitutes 64GB/s x 0.4 Flop/Byte = 25,6 GFlop/s.

Thus, all the possible variations are memory-bound

| GPU SMs | Peak throughput | Texe | Notes |
| --- | --- | --- | --- |
| 32 | 32 GFlops/s | 0,5s + 5s + 16GFlop / 25,6GFlop/s = 6,125s | Memory-bound |
| 64 | 64 GFlops/s | 6,125s | Memory-bound |
| 128 | 128 GFlops/s | 6,125s | Memory-bound |

Ultimately, the configuration using only a CMP with 8 cores should be chosen for this particular application.

1c)

If the CMP memory bandwidth is 32 GB/s, the peak transfer rate will be 16 GB/s x 0.4 Flop/Byte = 6.4GFlop/s. The system with 8 cores is now no longer memory-bound, but compute-bound, with an execution time of 0.5s + 16 GFlop / 4 GFlops/s = 4,5s.

The system with 8 cores will still be memory-bound to this higher bandwidth ie. 0.5s + 16 GFlop / 6,4 GFlops/s = 3s.

| Cores | Peak throughput | Texe | Notes |
| --- | --- | --- | --- |
| 4 | 2 GFlops/s | 0.5s + 16 Gflop / 2 GFlop/s = 8,5s | Compute-bound |
| 8 | 4 GFlops/s | 0.5s + 16 Gflop / 4 GFlop/s = 4.5s | Compute-bound |
| 16 | 8 GFlops/s | 0.5s + 16 Gflop / 6,4 GFlop/s = 3s | Memory-bound |

If the GPU memory bandwidth is increased to 256 GFlop/s, some of the cases will become compute-bound instead of memory-bound.

The effective bandwidth will be 128 GB/s

2a)

Assume cache lines in I state initially.

| Core | State z4 – C0 | State z5 – C0 | State z4 – C1 | State z5 – C1 |
| --- | --- | --- | --- | --- |
| C0(1) | I -> E (BusRd) | I | I | I |
| C0(2) |  | I->E (BusRd) |  |  |
| C1(1) | E->S (--) |  | I->S (BusRd) |  |
| C1(2) |  | E->S (--) |  | I->S (BusRd) |
| C0(3) |  | S->M (BusUpgr) |  | S->I (--) |
| C1(3) |  | M->I (Flush) |  | I->M (BusRdX) |
| C1(4) | S->I (--) |  | S->M (BusUpgr) |  |
| C0(4) | I->M (BusRdX) |  | M->I (Flush) |  |
| C0(5) |  | I->S (BusRd) |  | M->S(Flush) |
| C1(5) |  |  |  | S->S (--) |

2b)

This code seems to move data between different cache lines. In that case, MOESI is preferred as it has the capability to perform cache-to-cache transfers (owner to requester). In other words, it allows sharing of dirty data without memory write-back.

2c)